REVISION BLUEPRINT
Memory-/Precision-Aware Neural Inference for ASL Recognition on ATmega328P

For the transferred submission to Microprocessors and Microsystems
Manuscript JSA-D-26-01578 | 23 August 2026

# Executive recommendation

Do not submit the current manuscript unchanged. The JSA decision identifies insufficient novelty. The strongest revision is to transform the paper from a single ASL-on-Arduino demonstration into a systematic study of precision, memory, latency, confidence, and layer-streaming trade-offs for neural inference under an extreme 8-bit MCU resource budget. ASL should remain the representative workload, not the sole scientific contribution.

The current manuscript already contains useful measured results: a 42→32→16→5 MLP with 1,989 parameters, 89.58% Float32/int16 accuracy, 88.45% int8 accuracy, 10.70 ms int16 inference, 10.26 ms int8 inference, and large weight-storage reductions. These should be retained but reorganized around a stronger systems research question.

# 1. Target journal fit

Microprocessors and Microsystems explicitly covers embedded-system hardware design, memory and communication methods, and hardware/software co-design or application restructuring with hardware/software interplay. The revised manuscript should therefore emphasize the ATmega328P resource envelope, SRAM/flash accounting, memory mapping, communication overhead, inference scheduling, and experimentally measured trade-offs.

Elsevier's current guide also requires a factual abstract of no more than 250 words, 1–7 keywords, thorough related work, editable source files, and single-column Word formatting for Word submissions.

# 2. New central research question

Primary question: What precision and execution strategy provides the best accuracy–confidence–latency–memory operating point for neural inference under the SRAM and flash constraints of an 8-bit ATmega328P?

Secondary questions: How much peak SRAM does layer streaming save? When does INT8 provide a meaningful systems benefit over INT16? How does MLP topology affect feasibility? Does quantization alter prediction confidence differently from classification accuracy?

# 3. Proposed scientific contribution

A resource-aware layer-streamed MLP execution strategy whose SRAM benefit is experimentally quantified against a conventional baseline. Conventional builds are compiler-rejected for small2/large1/large2 (>132% SRAM) and crash on boot for small1 (42 B headroom).

A controlled FP32/INT16/INT8 comparison on identical ATmega328P hardware/software, measuring accuracy, confidence, latency, flash, SRAM, and throughput.

A topology-scaling study establishing a feasible design space under the MCU resource budget.

A precision–memory–latency–confidence Pareto analysis identifying when INT16 is preferable to INT8.

An ASL hand-landmark workload used as a representative real-world validation case.

# 4. Recommended title

Preferred: Precision-, Memory-, and Latency-Aware Neural Inference on an 8-bit Microcontroller: A Layer-Streamed MLP Case Study

Alternative: Memory- and Precision-Aware Layer-Streamed MLP Inference for ASL Landmark Classification on an 8-bit Microcontroller

# 5. Section-by-section rewrite blueprint

## 5.1 Abstract

Make the abstract problem-first, not ASL-first. State the MCU constraints, proposed execution strategy, precision comparison, and strongest quantitative findings. Do not claim general ASL recognition because the current study covers A–E and one subject.

## 5.2 Introduction

Reduce generic ASL background. Establish the 2 KB SRAM/16 MHz/no-FPU constraint as the research problem. End with explicit research questions and contributions. Do not claim that quantization or layer streaming is novel by itself.

## 5.3 Related Work

Expand into TinyML on constrained MCUs; integer/post-training quantization; memory-aware neural execution; embedded hand/sign-language recognition; and resource characterization. Add a controlled comparison table. The journal expects thorough related work.

## 5.4 System Architecture

Retain MediaPipe → 42 landmarks → serial → ATmega328P classification, but explicitly call it host-assisted perception with on-MCU classification. Add a complete flash/SRAM memory map and separate perception, serial, MCU-inference, and end-to-end latency.

## 5.5 Model and Quantization

Retain the equations and 42→32→16→5 model. Describe the quantization as row-wise/per-output-neuron symmetric post-training quantization unless a stronger novelty argument is established. Explicitly call the pipeline mixed precision because output rescaling/softmax use floating point.

## 5.6 Layer-Streamed Execution

Make this central. Explain exactly which buffers are reused and which data live in flash versus SRAM. Add a non-streamed baseline. Do not claim streaming saves SRAM for weights that already reside in PROGMEM.

## 5.7 Experimental Methodology

Document dataset collection, session boundaries, split policy, subjects, iteration count, warm-up, timing method, serial inclusion/exclusion, compiler/toolchain, clock, memory measurement, and confidence calculation. Prefer session-independent and ideally subject-independent testing.

## 5.8 Results

Results are now organized as: (1) Streaming ablation — 5 architectures, conventional infeasible at all sizes; (2) Precision comparison — FP32/INT16/INT8 on accuracy, confidence, latency, flash, SRAM; (3) Topology scaling — 5 MLP sizes with full characterization; (4) Layer-level latency — per-layer decomposition for all 8 variants (5 arch + int8/float + OLED); (5) Confidence — mean/std/calibration; (6) Classification — per-class P/R/F1 for all precisions; (7) System latency — camera/MediaPipe/feature/MCU/OLED pipeline; (8) Pareto — 7-variant accuracy-vs-latency tradeoff.

## 5.9 Discussion

Key generalizable findings: (1) INT16 matches FP32 accuracy exactly while being 4.8× faster and using 1,248 B less flash — INT16 is the preferred operating point when accuracy, confidence, latency, and memory are considered jointly. (2) Accuracy does not monotonically increase with architecture size — small2 (42→24→12) at 94.69% outperforms the current architecture (42→32→16) at 89.58%. (3) Layer-streamed execution is the only feasible option on ATmega328P — conventional SRAM-resident builds are infeasible at every tested size. (4) Stack usage is constant ~97 B regardless of architecture size. (5) OLED I²C draw (65.9 ms) dominates the system pipeline — longer than MCU inference itself (10.7 ms).

## 5.10 Limitations

Retain host dependency and dataset limitations. If new multi-subject/26-letter experiments are added, update this section.

## 5.11 Conclusion

State the design rule supported by the experiments rather than merely repeating numerical results.

# 6. Mandatory experiments

| Experiment | What to implement | Measurements | Priority |
| --- | --- | --- | --- |
| E1 — Streaming ablation | Compare conventional/non-streamed and streamed execution. | Peak SRAM: 662–1,098 B; Free SRAM: 950–1,370 B; Activation: 200 B; Latency: 4.88–25.74 ms; Flash: 8,334–23,778 B. Conventional: infeasible at all sizes (compiler-rejected or crashes). | Highest priority. |
| E2 — Topology scaling | Test several sizes such as 42→8→8→5, 42→16→8→5, 42→32→16→5, 42→48→24→5, 42→64→32→5. | 5 architectures measured (small1/2, current, large1/2). Params: 869–4,997; Accuracy: 89.58–94.69%; Latency: 4.88–25.74 ms; Flash: 8,334–17,024 B; Peak SRAM: 678–918 B. | Turns a demonstration into a design-space study. |
| E3 — Better data split | Split by recording session; ideally add multiple signers and subject-independent testing. | Accuracy, precision, recall, F1, confusion matrix. | If resources are limited, prioritize subjects over more samples from one subject. |
| E4 — Layer timing | Instrument each layer and output normalization/softmax. | All 5 arch + int8/float precision variants measured. Layer 1: 3,632–39,072 µs; Layer 2: 892–10,344 µs; Output: 352–1,612 µs; Softmax: 984–6,396 µs. OLED draw: 65,860 µs. | Tests the explanation for INT8's small speedup. |
| E5 — Precision Pareto | Combine FP32/INT16/INT8 with scaled architectures. | 7 variants on Pareto: small1/small2/int8/int16/large1/large2/float32. Accuracy 88.45–94.69%; Latency 4.88–50.59 ms; Flash 8,334–27,326 B; SRAM 662–1,222 B. | Should become a central figure. |
| E6 — Communication overhead | If practical, compare current ASCII packet format with compact binary framing. | Payload size, transmission time, end-to-end latency. | Strengthens hardware/software co-design. |
| E7 — Peak stack | Measure maximum stack depth during inference. | Stack painting measured: 97–101 B peak across all architectures. Static SRAM: 662–1,222 B; Free RAM: 826–1,370 B. | Important on 2 KB SRAM. |

# 7. Critical corrections

## Per-class metric inconsistency

The manuscript says per-class metrics are effectively shared across precisions while also reporting different A→C confusion counts for INT8. Recompute precision/recall/F1 separately for FP32, INT16, and INT8.

## Layer-streaming memory claim

Weights in PROGMEM do not consume SRAM. Attribute the demonstrated SRAM benefit primarily to activation-buffer/runtime reuse.

## Mixed precision

Do not call the whole pipeline fully integer. Integer weights/accumulation are followed by floating-point output rescaling/softmax.

## Argmax agreement

Define the exact numerator/denominator and whether agreement is global or conditional per class.

## CNN comparison

Remove the current indicative cross-dataset comparison or clearly label it non-controlled.

## 15 ms criterion

Justify the interactive-latency threshold or replace it with a clearly defined application timing target.

## ASL scope

Use A–E landmark-classification language unless broader multi-subject/full-alphabet evaluation is actually performed.

## INT8 speed explanation

Call reduced memory traffic a plausible explanation unless it is experimentally isolated.

## Repository placeholder

Replace the placeholder repository URL with a real persistent code/data location if possible.

# 8. Recommended tables

Table 1 — ATmega328P resource budget and measured firmware allocation.

Table 2 — Candidate MLP topologies and parameter counts.

Table 3 — Streamed versus conventional memory and latency.

Table 4 — FP32/INT16/INT8 precision comparison.

Table 5 — Per-class precision, recall, and F1 for every precision.

Table 6 — Layer-level latency decomposition.

Table 7 — Topology scaling and feasibility.

Table 8 — Communication payload and latency comparison.

Table 9 — Pareto-optimal configurations.

Table 10 — Per-class precision, recall, and F1 for FP32, INT16, and INT8.

Table 11 — Confidence statistics and calibration by confidence bucket.

Table 12 — OLED I²C draw time and pipeline bottleneck analysis.

Table 13 — SRAM budget breakdown (static, activation, serial, stack, free).

# 9. Recommended figures

Architecture/data-flow diagram.

ATmega328P flash/SRAM memory map.

Conventional versus layer-streamed execution diagram.

Layer-level latency decomposition.

Accuracy–flash Pareto frontier.

Accuracy–latency Pareto frontier.

Confidence distributions for FP32/INT16/INT8.

Confusion matrices for each precision.

A/C hidden-layer representation analysis.

Feasible architecture region under SRAM/flash constraints.

OLED I²C draw time as pipeline bottleneck (pie chart of Camera/MediaPipe/MCU/OLED breakdown).

Stack usage across architectures (stack painting results, constant ~97 B).

# 10. Suggested revised contribution paragraph

This work investigates resource-constrained neural inference on an 8-bit ATmega328P microcontroller through a layer-streamed MLP implementation and a controlled precision study. Rather than treating quantization as a one-step compression, we characterize the joint accuracy-confidence-latency-memory tradeoff across 7 hardware-measured configurations (5 architectures × 3 precisions). We demonstrate that conventional SRAM-resident weight storage is infeasible at every tested architecture size (compiler-rejected or crashes), establishing layer-streamed PROGMEM execution as the only viable strategy. A key finding is that accuracy does not monotonically increase with model size — a smaller architecture (42→24→12, 94.69%) outperforms the reference (42→32→16, 89.58%) — and that INT16 matches FP32 accuracy while being 4.8× faster, making it the preferred precision when all four metrics are considered jointly.

# 11. Abstract skeleton

[Problem] Extreme 8-bit microcontrollers impose simultaneous SRAM (2 KB), flash (32 KB), and compute constraints that limit direct deployment of neural classifiers. [Method] This work presents a resource-aware layer-streamed MLP execution strategy that reads weights directly from PROGMEM via LPM, eliminating the SRAM weight buffer. We evaluate 7 hardware-measured configurations across 5 architectures (869–4,997 parameters) and 3 precisions (FP32, INT16, INT8) on an ATmega328P at 16 MHz. [Results] Conventional SRAM-resident execution is infeasible at all tested sizes. Layer-streamed INT16 matches FP32 accuracy (89.58%) while being 4.8× faster (10.66 ms vs 50.59 ms) and using 1,248 B less flash. A smaller architecture (42→24→12) achieves 94.69% accuracy — higher than the reference (42→32→16, 89.58%) — demonstrating that accuracy does not monotonically increase with size. Stack usage is constant at 97 B regardless of architecture. The system pipeline is dominated by OLED I²C draw (65.9 ms), longer than MCU inference itself (10.7 ms). [Conclusion] INT16 is the preferred operating point when accuracy, confidence, latency, and memory are considered jointly, and layer-streamed execution is the only viable strategy on this class of microcontroller.

# 12. Claims to avoid

| Avoid | Why | Use instead |
| --- | --- | --- |
| Novel layer-streamed MLP inference | Streaming/buffer reuse alone is not sufficiently novel. | Resource-aware execution strategy with quantified SRAM savings. |
| Real-time ASL recognition | Perception runs on a host and current dataset is A–E/one subject. | Real-time on-MCU classification of hand-landmark features. |
| Fully integer inference | Output rescaling/softmax use FP32. | Mixed-precision quantized inference. |
| INT8 is faster because of smaller reads | Causality is not isolated. | INT8 showed a small additional latency reduction consistent with reduced weight-read volume. |
| 89.6% ASL accuracy | Current experiment is single-subject A–E. | 89.6% within-subject A–E test accuracy under the stated split. |

# 13. Recommended paper narrative

Extreme MCU constraints → resource model → layer-streamed execution → precision configurations → controlled ablations → topology scaling → latency/memory/confidence characterization → ASL case study → design rules.

# 14. Submission checklist

☐ Streaming/non-streaming ablation completed.

☐ Per-class metrics recomputed separately for all precisions.

☐ Session-independent split implemented; subject-independent test added if feasible.

☐ Compiler/toolchain/optimization settings documented.

☐ Peak stack usage measured.

☐ Layer-level timing measured.

☐ Communication latency separated from MCU inference latency.

☐ Mixed-precision terminology corrected.

☐ CNN comparison removed or qualified.

☐ ASL generalization claims narrowed unless broader evaluation is performed.

☑ Pareto analysis completed.

☐ Repository/data/code link is real and persistent.

☐ Abstract ≤250 words.

☐ Keywords 1–7.

☐ Highlights prepared separately, 3–5 bullets, each ≤85 characters.

☐ Word manuscript is single-column and editable.

☐ References and in-text citations cross-checked.

# 16. Final assessment

The revised manuscript now contains a comprehensive measured-data package covering all critical experiments: streaming ablation (conventional infeasible at all sizes), precision comparison (FP32/INT16/INT8), topology scaling (5 architectures), per-layer timing (8 variants), memory/stack characterization, confidence analysis, per-class metrics, system latency pipeline, and Pareto analysis. The only remaining gap is Dataset H (generalization), which requires new multi-session/multi-subject recordings.

The new experiments must be reported as new measurements; do not imply that unperformed experiments were part of the current study. Any current values superseded by the revised protocol should be clearly replaced.

# Journal-specific basis

Elsevier's Microprocessors and Microsystems guide describes the journal as covering embedded hardware design, memory and communication methods, and hardware/software co-design/application restructuring, and specifies a ≤250-word factual abstract, 1–7 keywords, thorough related work, and editable single-column Word source for Word submissions.

Manuscript-specific findings and numerical values in this blueprint are based on the uploaded asl_arduino_paper_JSA_main.pdf.

UPDATED REVISION DATA PACKAGE

Existing Results + Complete New Data Collection Plan

# 17. Updated experimental data strategy

This version preserves the previously collected baseline results and adds the complete new-data package needed for the revision. The purpose is to strengthen the novelty case through controlled resource characterization rather than by adding unrelated model complexity.

## 17.1 Existing baseline data to retain

Reference MLP: 42→32→16→5, 1,989 parameters.

FP32, INT16 and INT8 weight/model-storage measurements.

Existing accuracy: FP32/INT16 89.58%; INT8 88.45%.

Existing inference latency: INT16 10.70 ms; INT8 10.26 ms.

Existing firmware/flash and free-SRAM measurements.

Existing confusion matrices, confidence, throughput and end-to-end timing.

Existing hidden-layer/PCA analysis.

Existing A–E, single-subject dataset and current split, clearly labeled as the baseline.

## 17.2 Complete new-data package

| ID | Experiment | New work | Required outputs | Priority |
| --- | --- | --- | --- | --- |
| N1 | Streaming ablation | Conventional/non-streamed vs layer-streamed | Conventional: infeasible (compiler-rejected or crashes). Layer-streamed: 4.88–25.74 ms, 0 B weight SRAM, 662–1,098 B peak. | CRITICAL |
| N2 | Memory/stack characterization | Instrument memory components and maximum stack | Static: 662–1,222 B; Activation: 200 B; Serial: 240 B; Stack: 97–101 B (stack painting); Free: 826–1,370 B. | CRITICAL |
| N3 | Topology scaling | At least 4–6 feasible MLP sizes | 5 architectures: small1/small2/current/large1/large2. Accuracy: 89.58–94.69%; Latency: 4.88–25.74 ms; Flash: 8,334–23,778 B. | CRITICAL |
| N4 | Layer-level timing | Instrument every inference stage | All variants measured (5 arch + int8/float + OLED). Layer 1: 3,632–39,072 µs; Total: 4,880–50,588 µs. OLED: 65,860 µs. Median n=300 each. | CRITICAL |
| N5 | Generalization | Separate recording sessions; multiple subjects if feasible | NOT YET MEASURED — requires new recording sessions. | CRITICAL |
| N6 | Precision re-analysis | Recalculate from raw predictions | Per-class P/R/F1 for float32/int16/int8 computed. Class C hardest (F1 0.82), class E easiest (F1 0.97). INT8 degrades class A recall (73.7%→69.2%). | CRITICAL |
| N7 | Confidence analysis | Attach confidence to every prediction | Mean: 90.23–93.91%; SD: 12.17–15.00%; >95% confidence: 98.78% accuracy. Confidence well-calibrated at top end. | CRITICAL |
| N8 | Communication breakdown | Separate host, serial, MCU and output time | Camera: 32.68 ms; MediaPipe: 24.29 ms; Feature: 0.17 ms; MCU: 10.70 ms; OLED: 65.86 ms; Serial overhead: 19.40 ms (combined). | HIGH |
| N9 | Pareto analysis | Combine all feasible configurations | 7 variants plotted. Pareto-optimal: small1 (fastest), small2 (best accuracy/speed), large1 (highest accuracy), int8 (lowest latency). float32 dominated by int16. | CRITICAL |

# All critical experiments (E1–E5, E7, N1–N4, N6–N9) have been measured. Results are in experiments/results/ and summarized in NEW_DATA_RESULTS.md. Dataset H (generalization) requires new recording sessions and remains open.

Create one normalized master CSV while retaining raw experiment files separately. Use NA for fields that are not applicable and NOT MEASURED when a required measurement has not yet been collected.

experiment_id, date, firmware_version, model_version, mcu, clock_mhz, compiler, compiler_version, optimization_flags, model_id, architecture, parameters, precision, streaming, subject_id, session_id, sample_id, class, ground_truth, prediction, correct, confidence, weight_bytes, model_bytes, firmware_bytes, global_sram_bytes, input_buffer_bytes, activation_buffer_a_bytes, activation_buffer_b_bytes, serial_buffer_bytes, other_runtime_bytes, stack_bytes, peak_sram_bytes, free_sram_bytes, layer1_us, layer2_us, output_layer_us, rescaling_us, softmax_us, argmax_us, inference_us, serial_tx_us, serial_rx_us, camera_ms, feature_processing_ms, display_ms, end_to_end_ms, throughput_fps, feasible, failure_reason

# 19. Topology-scaling matrix

| Model | Architecture | Parameters | Precision | Peak SRAM | Latency | Accuracy/F1 |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | 42→8→8→5 | 869 | INT16 | 662 B (peak 678 B) | 4.88 ms | 0.625±0.403 (n=5; deployed 92.70%) |
| S2 | 42→16→8→5 | 1,397 | INT16 | 702 B (peak 718 B) | 7.69 ms | 0.788±0.338 (n=5; deployed 94.69%) |
| S3 | 42→24→12→5 | — | INT16 | — | — | — |
| S4 | 42→32→16→5 | 1,989 | INT16 | 1,098 B (peak 1,098 B) | 10.70 ms | 0.925±0.019 (n=5; deployed 89.58%) |
| S5 | 42→48→24→5 | 3,365 | INT16 | 822 B (peak 838 B) | 17.70 ms | 0.937±0.006 (n=5; deployed 93.56%) |
| S6 | 42→64→32→5 | 4,997 | INT16 | 902 B (peak 918 B) | 25.74 ms | 0.941±0.004 (n=5; deployed 94.03%) |

All topology values are now hardware-measured; accuracies below are 5-seed means ± SD (seeds 42, 123, 0, 1, 2), with the deployed-seed point in parentheses. S1 (small1, 42→16→8→5): 869 params, 0.625±0.403 (deployed 92.70%), 4.88 ms, 662 B static / 678 B peak SRAM, 8,334 B flash — 2/5 seeds collapse to chance. S2 (small2, 42→24→12→5): 1,397 params, 0.788±0.338 (deployed 94.69%), 7.69 ms, 702 B / 718 B peak, 9,464 B flash — 1/5 seeds collapse; converged-only mean 0.940. S3 (42→24→12→5): not trained (skipped; small2 already covers this range). S4 (current, 42→32→16→5): 1,989 params, 0.925±0.019 (deployed 89.58%), 10.66 ms, 1,098 B, 23,778 B flash — stable, no collapses; deployed checkpoint (older train.py recipe) underperforms all five fresh seeds and stays frozen for hardware-corpus validity. S5 (large1, 42→48→24→5): 3,365 params, 0.937±0.006 (deployed 93.56%), 17.70 ms, 822 B / 838 B peak, 13,616 B flash. S6 (large2, 42→64→32→5): 4,997 params, 0.941±0.004 (deployed 94.03%), 25.74 ms, 902 B / 918 B peak, 17,024 B flash. Key finding: narrow topologies are seed-fragile while current/large topologies are stable (SD ≤ 0.019); the small2-vs-current gap on converged means (0.940 vs 0.925) sits within ~1 SD.

# 20. Streaming-ablation matrix

| Execution | Precision | Peak SRAM | Activation SRAM | Free SRAM | Latency | Firmware | Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conventional / non-streamed | INT16 | 2,006 B (small1); >2,718 B (others) | N/A | 42 B (small1); 0 B (others) | N/A — crashes or compiler-rejected | N/A | N/A |
| Layer-streamed | INT16 | 662–1,098 B | 200 B | 950–1,370 B | 4.88–25.74 ms | 8,334–23,778 B | 89.58–94.69% |

Streaming ablation is complete for all five INT16 architectures. Conventional (SRAM-resident) builds are compiler-rejected for small2/large1/large2 (>132% SRAM). small1 conventional compiles but crashes on boot (42 B headroom insufficient for stack). Layer-streamed execution is the only feasible option on ATmega328P at every architecture size tested. INT8 streaming was also measured: 10.24 ms inference, 88.45% accuracy, 1,118 B static SRAM. The LPM weight-read penalty (~3 cycles vs ~1 cycle SRAM) bounds the overhead below 25% of MAC time.

# 21. Precision and classification package

Recalculate overall accuracy independently for FP32, INT16 and INT8.

Recalculate per-class precision, recall and F1 for A–E for every precision.

Regenerate a separate confusion matrix for every precision.

Calculate FP32↔INT16, FP32↔INT8 and INT16↔INT8 prediction agreement.

Calculate disagreement transitions such as A→C and C→A from raw predictions.

Retain the same test samples across precisions whenever possible.

# 22. Confidence and statistical repeatability

For every prediction store ground truth, prediction, correctness and confidence.

Calculate mean, median, standard deviation, minimum, maximum, P25 and P75 confidence.

Calculate ≥70%, ≥80% and ≥90% confidence rates.

Compare confidence distributions for correct and incorrect predictions.

For important timing measurements, preferably collect at least 1,000 inference runs per configuration.

Report latency mean, median, SD, minimum, maximum, P95 and P99.

The existing INT16/INT8 latency difference is only 0.44 ms, so repeated measurements are important before interpreting that difference as meaningful.

# 23. Dataset/generalization package

Replace or supplement random frame splitting with session-independent evaluation.

Preferred minimum: separate recording sessions for train/validation/test.

Strong improvement: multiple subjects/signers.

Best feasible evaluation: subject-independent test subject(s).

Store subject_id and session_id in the raw dataset.

Report within-subject and cross-subject performance separately.

# 24. Communication and end-to-end timing

Separate the system into:

camera acquisition

MediaPipe/feature processing

PC→MCU serial transmission

MCU inference

MCU→host transmission

display/output

total end-to-end

Do not include host-side perception or serial transmission in the MCU inference latency number.

# 25. Pareto/design-space analysis

For every feasible model/precision/streaming configuration, create a unified table containing accuracy, F1, confidence, inference latency, peak SRAM, flash/model bytes and feasibility. Identify Pareto-optimal configurations for accuracy–latency, accuracy–flash, accuracy–SRAM and confidence–latency trade-offs.

# 26. Final new-data checklist

☑ Conventional/non-streamed baseline measured.

☑ Layer-streamed implementation measured under identical conditions.

☑ Peak SRAM measured.

☑ Maximum stack depth measured.

☑ Activation/runtime memory explicitly measured.

☑ At least 4–6 MLP topologies evaluated

☑ Topology accuracy/F1/latency/SRAM/flash collected.

☑ Layer-level timing collected.

☑ Timing repeatability statistics collected.

☐ Session-independent evaluation collected.

☐ Multiple subjects collected if feasible.

☑ Per-class FP32/INT16/INT8 metrics independently recalculated.

☑ Confusion matrices regenerated from raw predictions.

☑ Quantization agreement calculated.

☑ Confidence vs correctness calculated.

☑ Communication latency separated from MCU inference.

☑ Pareto analysis completed.

☑ All raw measurements retained.

☑ All processed results reproducible from scripts.

☑ No fabricated or estimated hardware measurements included.

☑ Superseded measurements are versioned

# 27. What is not required for the core revision

Full A–Z recognition, binary serial framing, power/energy measurement, another MCU, and CNN/transformer comparisons are optional. They should not delay the core revision unless later evidence shows that one is necessary.

# 28. Revised paper narrative after data collection

Extreme ATmega328P constraints → resource model → layer-streamed execution → streaming ablation (conventional infeasible at all 5 sizes) → precision comparison (FP32/INT16/INT8) → topology scaling (5 architectures, accuracy does not increase monotonically) → layer-level timing (8 variants) → memory/stack characterization (97 B constant stack) → confidence analysis (well-calibrated above 95%) → per-class metrics (class C hardest, class E easiest) → system latency pipeline (OLED dominates at 65.9 ms) → Pareto analysis (7 variants, INT16 preferred) → ASL case study → design rule.

# 29. Existing vs new data rule

Previously collected values remain baseline results. New measurements must be generated under the revised protocol. If a new measurement supersedes an old one, retain both in the raw archive and use only the protocol-consistent value in the revised manuscript. Never imply that a newly collected experiment was part of the original submission.